# Getting Started

- Start with the motherboard oriented as in the manual

# Install the CPU

- Lift up the lever that locks in the CPU.

- Install the CPU with the gold arrow in the opposite corner of the base of the lever that locks in the CPU.  It should drop into position.

- Lock the CPU down by pushing down on the lever.

# Provide Power to the CPU

- Connect the 4-pin power line from the power supply into JPWR2.

# Install Heat Sink and CPU fan

- Attach the heat sink and CPU fan to the top of the CPU.

- Connect the power line on the CPU fan to CPUFAN on the motherboard.

# Install RAM

- Install DIMM into DIMM1 (the slot closest to the CPU)

# Install hard drive

- Connect hard drive to SATA1.

- Connect an SATA power line to the hard drive.

# Connect speaker and Power Button

- Install the speaker on pins 2, 4, 6 and 8 on JFP2 (JFP2 may look different depending on the motherboard).

- Install the power button on pins 6 and 8 on JFP1.

# Main Power

- Connect the 24-pin plug from the power supply to the motherboard.

# Keyboard, Monitor and Mouse

- Plug in the keyboard, monitor and mouse and boot up.

A75MA-E35 Motherboard Layout

A78M-E35 Motherboard Layout